Exp no: 8

Cloud Application using Adafruit IO Cloud

### Aim:

To connect a Raspberry Pi 4 with a DHT11 Temperature & Humidity Sensor and send real-time sensor data to Adafruit IO Cloud for monitoring and visualization.

## Apparatus / Requirements

- Hardware

  - Raspberry Pi 4 with Raspbian OS installed

  - DHT11 Temperature & Humidity Sensor

  - Breadboard and Jumper Wires

  - Internet connection

- Software

  - Python 3.x

  - Adafruit IO account (https://io.adafruit.com)

  - Python libraries (install using terminal): pip3 install adafruit-io adafruit-circuitpython-dht adafruit-blinka

## Theory

In IoT, cloud applications enable devices to collect and transmit data for remote access and visualization.
 Adafruit IO is an IoT cloud platform that provides:

- Feeds → storage for sensor data

- Dashboards → visualization (charts, gauges)

- Triggers & Automation → notifications and actions

In this experiment, the Raspberry Pi 4 reads temperature and humidity from the DHT11 sensor and sends it to Adafruit IO feeds using the Adafruit IO Python client. The data is displayed on an Adafruit Dashboard in real-time.

## Circuit Connections:

- DHT11 VCC → 3.3V (Pin 1)

- DHT11 GND → GND (Pin 6)

- DHT11 DATA → GPIO4 (Pin 7)

## Procedure

- Create Adafruit IO account and note down:

  - Username

  - AIO Key

- Create two feeds:

  - temperature

  - humidity

- Write the Python Program on Raspberry Pi

- Save the program as dht_adafruit.py.

- Run it with:
 python3 dht_adafruit.py

- Go to Adafruit IO Dashboard, add Line Chart / Gauge blocks, and select the temperature and humidity feeds.

- Observe real-time updates on the dashboard.

## Result

Successfully read real-time temperature and humidity from the DHT11 sensor using Raspberry Pi 4 and uploaded the data to Adafruit IO Cloud for visualization.

import time

import board

import adafruit_dht

from Adafruit_IO import Client

# Adafruit IO credentials

aio = Client("YOUR_USERNAME", "YOUR_AIO_KEY")

# Initialize DHT11 sensor on GPIO4

dht = adafruit_dht.DHT11(board.D4)

while True:

try:

# Read data from DHT11

temp = dht.temperature

hum = dht.humidity

# Send data to Adafruit IO feeds

aio.send("temperature", temp)

aio.send("humidity", hum)

# Print on terminal

print(f"Sent -> Temperature: {temp} °C | Humidity: {hum} %")

except Exception as e:

print("Error:", e)

# Delay for 10 seconds

time.sleep(10)